Class: TYIT	Sem: V	Roll No.:TYIT16	Date :04/07/26 Course Name: IOT				Page no:

Practical Number:- 1A

AIM: A demonstrate traffic light using led and Arduino Uno on a simulation software. STEPS:

- On the tinkercad ,click on the create circuit .

- Search ardiuno uno r3,breadboad,and leds and resistors.

- Leds are placed in breadboad like red led in d3,yellow led in d13,and led in 24 respectively.

- Resistors are placed parallel to led, one is connected to led and other one on GND(negative).

- On ardiuno in digital side the wire is placed in 3.3v and it is connected to the positive of the breadboard and GND is connected to breadboad GND(negative).

- At the digital side, 3 wire are pinned in 4,-3, and 2 and the other end is connected were led and resistor are pinned.

- After connection and code we can start the simulation

CODE:

int red = 4;

int yellow = 3; int green = 2;

void setup()

{

pinMode(red,OUTPUT); pinMode(yellow,OUTPUT); pinMode(green,OUTPUT);

}

void loop()

{

digitalWrite(red, HIGH); digitalWrite(yellow, LOW); digitalWrite(green, LOW); delay(500); digitalWrite(red, LOW); digitalWrite(yellow, HIGH); digitalWrite(green, LOW); delay(500); digitalWrite(red, LOW); digitalWrite(yellow, LOW); digitalWrite(green, HIGH); delay(500);

}

Class: TYIT	Sem: V	Roll No.:TYIT16	Date :04/07/26 Course Name: IOT				Page no:

Practical Number:- 1A

Class: TYIT	Sem: V	Roll No.:TYIT16	Date :04/07/26 Course Name: IOT				Page no:

Practical Number:- 1A

OUTPUT: CIRCUIT DIAGRAM

Class:TYIT	Sem: V	Roll No.: TYIT16	Date :09/07/26

Course Name: Internet Of Things	Page no:

Practical Number:- 1b

# AIM:Demonstrating the use of Pushbutton. Steps:

- On the Tinkercad,click on the create circuit.

- Search Arduino uno r3,Breadboard,Leds,Resistors and Pushbutton.

- Then place the LED,resistor and the pushbutton on the breadboard.

- Then connect the LED with the Arduino (in d13) Digital side.

- Also connect the Pushbutton with Arduino (in -3) Digital side.

- Then the cathode(positive) is connected to the 3.3V on the Analog side.

- The Anode(negative) is connected to the GND of the Analog side.

- Also from the Breadboard connect the Cathode (positive) to the pushbutton.

- After connection and code we can start the simulation.

Code:

int red = 13;

int buttonstate = 0; int pushbutton= 3;

void setup()

{

pinMode(pushbutton,INPUT); pinMode(red, OUTPUT);

}

void loop()

{

buttonstate = digitalRead(3); if(buttonstate == HIGH)

{

digitalWrite(13, HIGH);

}

else{

digitalWrite(13, LOW);

}

delay(10);

}

Class:TYIT	Sem: V	Roll No.: TYIT16	Date :09/07/26

Course Name: Internet Of Things	Page no:

Practical Number:- 1b

Output:

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026 Course Name:IOT			 Page no:

Practical Number:-2

AIM(a):Interfacing 16x2 LCD Display with Arduino Board to Display text with scrolling effect. Code:

#include <LiquidCrystal.h>

// RS, E, D4,D5, D6, D7

LiquidCrystal lcd(8, 7, 6, 5, 4, 3);

void setup() { lcd.begin(16, 2);

}

void loop() {

for(int i = 1;i <= 60; i++) {

//First	row

lcd.setCursor(0,	0); lcd.print("Class of TYIT:");

//Create Roll Number

String roll = "TYIT" + String(i);

//Add spaces before and after for scrolling effect String scrollText= "	" + roll + "	";

//Scroll the roll number across the second row

for (int pos = 0; pos <= scrollText.length() - 16; pos++) { lcd.setCursor(0, 1);	lcd.print(scrollText.substring(pos, pos + 16));	delay(50);

}

delay(500);

}

}

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026 Course Name:IOT			 Page no:

Practical Number:-2

Output:

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026 Course Name:IOT			 Page no:

Practical Number:-2

AIM(b): 2b.to Interfacing 16x2 LCD Display with Arduino Board Display a list of items. CODE:

#include <LiquidCrystal.h>

// RS, E, D4, D5, D6, D7

LiquidCrystal lcd(8, 7, 6, 5, 4, 3);

String items[] = { “Monsoon”, “Winter”,

“Autumn”,

“Spring”, “Summer”

};

Void setup() { Lcd.begin(16, 2);

}

Void loop() {

For (int I = 0; I < 5; i++) { Lcd.clear();

// First Row Lcd.setCursor(0, 0); Lcd.print(“List Seasons”);

// Second Row Lcd.setCursor(0, 1); Lcd.print(items[i]);

Delay(2000);

}

}

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026 Course Name:IOT			 Page no:

Practical Number:-2

OUTPUT:

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026

Course Name:IOT	Page no:

Practical Number:-2

AIM 2(c): Interfacing 16x2 LCD and pushbutton with Arduino.

Code:

#include <LiquidCrystal.h> LiquidCrystal lcd(8, 7, 6, 5, 4, 3); const int buttonPin = 2;

bool lcdState = false;

bool lastButtonState = HIGH;

void setup() { lcd.begin(16, 2);

pinMode(buttonPin, INPUT_PULLUP); lcd.noDisplay();	// LCD starts OFF

}

void loop() {

bool currentButtonState = digitalRead(buttonPin);

if (lastButtonState == HIGH && currentButtonState == LOW) { lcdState = !lcdState;	// Toggle LCD state

if (lcdState) { lcd.display(); lcd.clear(); lcd.setCursor(0, 0); lcd.print("LCD ON!"); lcd.setCursor(0, 1);

lcd.print("Hello TYIT16");

} else { lcd.noDisplay();

}

delay(200);  // Debounce

}

lastButtonState = currentButtonState;

}

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :21/07/2026

Course Name:IOT	Page no:

Practical Number:-2

Output:

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 01/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 2D

Aim: Design and implement a small doorbell using a push button in tinkercad when the button is pressed activate a buzzer, light an LED, and display "Doorbell Pressed" on the LCD to display to simulate a smartphone notification

Steps:

- Open Tinkercad and create a new blank Circuit.

- Drag an Arduino Uno R3, a breadboard, a 16×2 LCD (parallel interface), a 10 kΩ potentiometer, a 220 Ω resistor, a LED, a push button, and a piezo buzzer onto the workspace.

- Place the 16×2 LCD on the breadboard so that its 16 pins are inserted into separate rows.

- Connect the Arduino power pins:

  - Arduino 5V → Breadboard positive (+) rail

  - Arduino GND → Breadboard negative (−) rail

- 5. Power the LCD:

  - LCD Pin 1 (VSS) → GND rail

  - LCD Pin 2 (VDD) → +5V rail

  - LCD Pin 15 (LED+) → +5V through a 220 Ω resistor

  - LCD Pin 16 (LED−) → GND rail

- Connect the contrast control using the 10 kΩ potentiometer:

  - One outer terminal → +5V rail

  - Other outer terminal → GND rail

  - Middle terminal (wiper) → LCD Pin 3 (VO)

- Connect the LCD control pins to the Arduino:

  - LCD Pin 4 (RS) → Arduino Digital Pin 12

  - LCD Pin 5 (RW) → GND

  - LCD Pin 6 (E) → Arduino Digital Pin 11

- Connect the LCD data pins in 4-bit mode:

  - LCD Pin 11 (D4) → Arduino Digital Pin 5

  - LCD Pin 12 (D5) → Arduino Digital Pin 4

  - LCD Pin 13 (D6) → Arduino Digital Pin 3

  - LCD Pin 14 (D7) → Arduino Digital Pin 2

  - Leave LCD Pins 7–10 (D0–D3) unconnected.

- Connect the LED:

  - Arduino digital output pin → 330 Ω resistor

  - Resistor → LED anode (+)

  - LED cathode (−) → GND

- Connect the push button:

  - Place the push button across the center gap of the breadboard.

  - One side → Arduino digital input pin

  - Other side → GND

  - Configure the input as INPUT_PULLUP in the Arduino program if using this connection.

- Connect the piezo buzzer:

  - Buzzer positive (+) → Arduino digital output pin

  - Buzzer negative (−) → GND

- Write and upload the Arduino program using the LiquidCrystal library. For the LCD connections above, use: LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

- Start the simulation in Tinkercad.

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 01/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 2D

- Rotate the 10 kΩ potentiometer until the LCD characters become visible.

- Test the circuit by pressing the push button and checking the LED and buzzer response programmed in the Arduino sketch.

- . Verify that the LCD displays the required text or values and that all components operate correctly.

Code:-

#include <LiquidCrystal.h>

// LCD Pins: RS, E, D4, D5, D6, D7

LiquidCrystal lcd(8, 7, 6, 5, 4, 3);

// Component Pins const int buttonPin = 2; const int ledPin = 9;

const int buzzerPin = 10;

void setup() {

pinMode(buttonPin, INPUT_PULLUP); pinMode(ledPin, OUTPUT); pinMode(buzzerPin, OUTPUT);

digitalWrite(ledPin, LOW); noTone(buzzerPin);

lcd.begin(16, 2);

lcd.clear(); lcd.setCursor(0, 0);

lcd.print("Smart Doorbell"); lcd.setCursor(0, 1); lcd.print("Waiting...");

}

void loop() {

// Button pressed (INPUT_PULLUP -> LOW) if (digitalRead(buttonPin) == LOW) {

// Turn ON LED digitalWrite(ledPin, HIGH);

// Turn ON buzzer tone(buzzerPin, 1000);

// Display notification lcd.clear(); lcd.setCursor(0, 0);

lcd.print("Doorbell TYIT16"); lcd.setCursor(0, 1);

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 01/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 2D

lcd.print("Pressed");

// Keep everything ON for 3 seconds delay(3000);

// Turn OFF buzzer and LED noTone(buzzerPin); digitalWrite(ledPin, LOW);

// Return to waiting screen lcd.clear(); lcd.setCursor(0, 0);

lcd.print("Smart Doorbell"); lcd.setCursor(0, 1); lcd.print("Waiting...");

// Wait until button is released

while (digitalRead(buttonPin) == LOW);

delay(200);  // Debounce

}

}

Circuit Diagram:

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 01/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 2D Output:-

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 08/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 3A

Aim: Design and implement a parking assistance system using an ultrasonic sensor. Measure the distance to an obstacle and activate a buzzer when the object is too close.

Steps:

Open Tinkercad and create a new blank Circuit.

- Drag an Arduino Uno R3, a breadboard, an HC-SR04 ultrasonic sensor, and a piezo

- buzzer onto the workspace.

- Place the HC-SR04 ultrasonic sensor on the breadboard/workspace so that its four pins are accessible.

- Connect the Arduino power pins:

  - Arduino 5V → Breadboard positive (+) rail

  - Arduino GND → Breadboard negative (−) rail

- Connect the HC-SR04 power pins:

  - HC-SR04 VCC → +5V rail

  - HC-SR04 GND → GND rail

- Connect the HC-SR04 control pins to the Arduino:

  - HC-SR04 TRIG → Arduino Digital Pin 7

  - HC-SR04 ECHO → Arduino Digital Pin 6

- Place the piezo buzzer on the breadboard and connect it:

  - Buzzer positive (+) → Arduino Digital Pin 8

  - Buzzer negative (−) → GND rail

- Verify all connections:

  - 5V supplies power to the HC-SR04.

  - GND is shared by the Arduino, sensor, and buzzer.

  - D7 is used for the ultrasonic sensor TRIG pin.

  - D6 is used for the ultrasonic sensor ECHO pin.

  - D8 controls the buzzer.

- Write and upload the Arduino program to measure the distance using the HC-SR04 and

- control the buzzer.

Code:-

int buzPin=7; int trigPin=8; int echoPin = 4;

float speed = 0.0347; float dist;

float pingTime; int buzNear=20; int buzHigh=50; int buzMid=130; int buzFar=600;

int delayFar = 260;

void setup()

{

Serial.begin(9600);

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 08/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 3A

pinMode(buzPin, OUTPUT); pinMode(trigPin , OUTPUT); pinMode(echoPin , INPUT);

}

void loop()

{

digitalWrite(trigPin, LOW); delayMicroseconds(20); digitalWrite(trigPin , HIGH); delayMicroseconds(10); digitalWrite(trigPin , LOW); pingTime = pulseIn(echoPin , HIGH); dist = (speed*pingTime*0.5);

Serial.print("Practical performed by Namrata TYIT16:"); Serial.println(dist);

if(dist<=10.0)

{

digitalWrite(buzPin, HIGH); delay(20); digitalWrite(buzPin , LOW); delay(20);

}

else if(dist<=3.0 && dist>10.0)

{

digitalWrite(buzPin , HIGH); delay(buzHigh); digitalWrite(buzPin , LOW); delay(buzHigh);

}

else if((dist>30.0) && (dist<60.0))

{

digitalWrite(buzPin , HIGH); delay(buzMid); digitalWrite(buzPin , LOW); delay(buzMid);

}

else if ((dist>=60.0 && dist<120.0))

{

digitalWrite(buzPin , HIGH); delay(buzFar); digitalWrite(buzPin , LOW); delay(delayFar);

}

}

Class :- TYIT	Sem :- V	Roll No. :- TYIT16	Date :- 08/08/2026 Course Name :-Internet Of Things			Page no :-

Practical Number :- 3A Circuit Diagram:-

Output:-

| BHAVAN’S CO Class: TYIT | LLEGE A PRACT Sem: V | UTONOMOUS, ANDHE ICAL JOURNAL Roll No.: TYIT16 | RI-WEST Date :19/08/26 |
| --- | --- | --- | --- |
| Course Name:NGT |  |  | Page no: |

Practical Number:-IOT

AIM: Design and implement a temperature monitoring system using the TMP36 sensor. Display the temperature on the Serial Monitor and glow an LED whenever the temperature exceeds a predefined threshold.

STEPS:

- Connect the TMP36 temperature sensor to A0.

- Connect the Red LED to pin 10.

- Connect the Blue LED to pin 12.

- Start the Arduino and read the temperature from the sensor.

- Convert the sensor value into temperature in °C.

- Show the temperature on the Serial Monitor.

- If temperature is above 50°C, turn Red LED ON.

- If temperature is below 0°C, turn Blue LED ON.

- If temperature is between 0°C and 50°C, keep both LEDs OFF.

- Repeat the process every 500 milliseconds.

CODE:

// Temperature-Controlled LED System const int tempPin = A0; const

int redLED = 10;

const int blueLED = 12;

void setup()

{

pinMode(redLED, OUTPUT); pinMode(blueLED, OUTPUT);

Serial.begin(9600);

}

void loop()

{

// Read the sensor value (0-1023) int sensorValue = analogRead(tempPin);

// Convert the reading to voltage float voltage = sensorValue * (5.0 / 1023.0);

// Convert the voltage to temperature (TMP36) float temperature = (voltage - 0.5) * 100.0;

// Display the temperature in the Serial Monitor Serial.println("Practical Performed by Bhumi "); Serial.print("Current Temperature is: "); Serial.print(temperature);

Serial.println(" °C");

BHAVAN’S COLLEGE AUTONOMOUS, ANDHERI-WEST

PRACTICAL JOURNAL

Class: TYIT	Sem: V	Roll No.: TYIT16	Date :19/08/26 Course Name:IOT				Page no:

Practical Number:-

Serial.println();

// Temperature above 50°C → Red LED ON if (temperature > 50)

{

digitalWrite(redLED, HIGH); digitalWrite(blueLED, LOW);

}

// Temperature below 0°C → Blue LED ON else if (temperature < 0)

{

digitalWrite(blueLED, HIGH); digitalWrite(redLED, LOW);

}

// Temperature between 0°C and 50°C → Both LEDs OFF else

{

digitalWrite(redLED, LOW); digitalWrite(blueLED, LOW);

}

delay(500);

Class: TYIT	Sem:V	Roll No.: TYIT16	Date : 02/09/2026

Course Name:IOT	Page no:

Practical Number:-

AIM: Design and implement a gas leakage monitoring system using an MQ-2 gas sensor. If gas concentration exceeds a threshold, switch ON an LED, activate a buzzer, and display a warning message on the Serial Monitor.

STEPS:

- Connect the MQ-2 gas/smoke sensor to analog pin A3.

- Connect 5 LEDs to pins 2, 3, 4, 5, and 6.

- Connect the buzzer to pin 12.

- Start the Arduino and read the value from the MQ-2 sensor.

- Display the sensor value on the Serial Monitor.

- If the value is below 970, all LEDs and the buzzer remain OFF.

- If the value is 970–978, 1 LED turns ON.

- If the value is 979–986, 2 LEDs turn ON.

- If the value is 987–994, 3 LEDs turn ON.

- If the value is 995–1003, 4 LEDs turn ON.

- If the value is 1004 or above, all 5 LEDs and the buzzer turn ON.

- The system repeats the process every 200 milliseconds.

Code:

// MQ-2 Smoke/Gas Detection System

// Tinkercad sensor range: approximately 970 to 1013 const int gasSensor = A3;

const int led1 = 2; const int led2 = 3; const int led3 = 4; const int led4 = 5; const int led5 = 6;

const int buzzer = 12; int gasValue;

void setup()

{

pinMode(led1, OUTPUT); pinMode(led2, OUTPUT); pinMode(led3, OUTPUT); pinMode(led4, OUTPUT); pinMode(led5, OUTPUT);

pinMode(buzzer, OUTPUT);

Class: TYIT	Sem:V	Roll No.: TYIT16	Date : 02/09/2026

Course Name:IOT	Page no:

Practical Number:-

Serial.begin(9600);

}

void loop()

{

// Read MQ-2 sensor

gasValue = analogRead(gasSensor);

Serial.print("Smoke Detected at radius: "); Serial.println(gasValue);

Serial.println("Practical Performed by Bhumi Pandey"); Serial.println();

// Turn OFF all LEDs and buzzer digitalWrite(led1, LOW); digitalWrite(led2, LOW); digitalWrite(led3, LOW); digitalWrite(led4, LOW); digitalWrite(led5, LOW); digitalWrite(buzzer, LOW);

// Very low smoke if (gasValue < 970)

{

// All LEDs OFF

}

// Stage 1

else if (gasValue < 979)

{

digitalWrite(led1, HIGH);

}

// Stage 2

else if (gasValue < 987)

{

digitalWrite(led1, HIGH); digitalWrite(led2, HIGH);

}

// Stage 3

else if (gasValue < 995)

{

digitalWrite(led1, HIGH);

Class: TYIT	Sem:V	Roll No.: TYIT16	Date : 02/09/2026

Course Name:IOT	Page no:

Practical Number:-

digitalWrite(led2, HIGH); digitalWrite(led3, HIGH);

}

// Stage 4

else if (gasValue < 1004)

{

digitalWrite(led1, HIGH); digitalWrite(led2, HIGH); digitalWrite(led3, HIGH); digitalWrite(led4, HIGH);

}

// Maximum smoke/gas concentration else

{

digitalWrite(led1, HIGH); digitalWrite(led2, HIGH); digitalWrite(led3, HIGH); digitalWrite(led4, HIGH); digitalWrite(led5, HIGH);

digitalWrite(buzzer, HIGH);

}

delay(200);

}

Class: TYIT	Sem:V	Roll No.: TYIT16	Date : 02/09/2026

Course Name:IOT	Page no:

Practical Number:-

Output:

| BHAVAN Class: TYIT | ’S COLL Sem: V | EGE AUTONOMOUS, AND PRACTICAL JOURNAL Roll No.: TYIT16 | HERI-WEST Date:09/09/2026 |
| --- | --- | --- | --- |
| Course Name: IOT |  |  | Page no: |

Practical Number: - 06

AIM: Interface Raspberry Pi with LED Module to demonstrate LED pattern. Setup:

  - Connect Raspberry Pi with Keyboard, Mouse, Monitor and Ethernet.

  - Take eight jumper wires [female to female] and connects them to LED and GPIO pins. 3] Take one jumper wire and connect it to the ground.

Code:

import RPi.GPIO as GPIO import time

# Use physical pin numbering GPIO.setmode(GPIO.BOARD)

# LED connections using physical pin numbers leds

= [37, 35, 33, 31, 29, 32, 36, 38]

# Set LED pins as output for led in leds:

GPIO.setup(led, GPIO.OUT) GPIO.output(led, GPIO.LOW)

try:	while True:

# LED pattern: 1 → 2 → 3 → 4 for led in leds:

GPIO.output(led, GPIO.HIGH) time.sleep(0.5)

GPIO.output(led, GPIO.LOW)

# LED pattern: 4 → 3 → 2 → 1 for led in reversed(leds): GPIO.output(led, GPIO.HIGH) time.sleep(0.5)

GPIO.output(led, GPIO.LOW)

except KeyboardInterrupt:	print("Program stopped")

finally:

GPIO.cleanup()

| Class: TYIT | Sem: V | Roll No.: TYIT16 | Date:09/09/2026 |
| --- | --- | --- | --- |
| Course Name: IOT |  |  | Page no: |

BHAVAN’S COLLEGE AUTONOMOUS, ANDHERI-WEST

PRACTICAL JOURNAL

Practical Number: - 06

Circuit:

Output:

Teacher’s Signature

BHAVAN’S COLLEGE AUTONOMOUS, ANDHERI-WEST

PRACTICAL JOURNAL

Class: TYIT(NEP)	Sem: V	Roll No.: TYIT16	Date: 09/09/2026

Course Name: Internet of Things	Page no: Practical Number:- 7

# AIM: DISPLAYING TIME OVER A 4-DIGIT SEVEN SEGMENT DISPLAY USING RASPBERRY PI.

- STEPS:

    - Connect the Raspberry Pi to a keyboard, mouse, and Ethernet cable.

    - Connect the monitor to the Raspberry Pi using an HDMI cable.

    - Connect the GND pin of the TM1637 display module to a GND pin of the Raspberry Pi using a female-to-female jumper wire.

    - Connect the VCC/Power pin of the TM1637 display module to the appropriate power pin of the Raspberry Pi using a female-to-female jumper wire.

    - Connect the DIO pin of the TM1637 module to physical pin 18 (GPIO 24) of the Raspberry Pi.

    - Connect the CLK pin of the TM1637 module to physical pin 16 (GPIO 23) of the Raspberry Pi.

    - Connect the Raspberry Pi to a power supply using a USB power cable.

    - Open Thonny Python IDE on the Raspberry Pi.

    - Type or paste the required Python program into Thonny.

    - Make sure that tm1637.py and pract7.py are saved in the same folder/location.

    - Save the program as pract7.py.

    - Click Run in Thonny to execute the program.

    - Observe the output on the TM1637 display module.

# CODE:

## pract7.py

import sys import time import datetime import RP1.GPIO as GPIO import tm1637

#CLK GPI023 (Pin 16) #D10 GP1024 (Pin 18)

Display tm1637.TM1637(23, 24, tm1637.BRIGHT_TYPICAL)

Display.Clear() Display.SetBrightness(1)

while(True):

now = datetime.datetime.now() hour

= now.hour

Teacher’s Signature

BHAVAN’S COLLEGE AUTONOMOUS, ANDHERI-WEST PRACTICAL JOURNAL

Class: TYIT(NEP)	Sem: V	Roll No.: TYIT16	Date: 09/09/2026

Course Name: Internet of Things	Page no:

Practical Number:- 7 minute = now.minute second = now.second currenttime = [int(hour/10), hour % 10, int(minute/10), minute % 10 ] Display. Show(currenttime)

Display. ShowDoublepoint(second % 2) time.sleep(1)

# CIRCUIT DIAGRAM:

### OUTPUT:

Teacher’s Signature